WELDING ARM CONTROL PROJECT

Simple Setup, Uploading and Operating Guide

This guide explains how to upload the Arduino program, install the Android app, connect the HC-05 Bluetooth module, and test the robotic arm.

# 1. Files Required

Robotic_Arm.ino — Arduino program that controls the servo motors.

Welding Arm Control.apk — Android app used to send movement commands.

# 2. How the Project Works

| 1. Android app | Sends a movement command. |
| --- | --- |
| 2. HC-05 Bluetooth | Receives the command wirelessly. |
| 3. Arduino Uno | Reads the command and controls the servos. |
| 4. Servo motors | Move the base, shoulder and elbow of the arm. |

# 3. Hardware Wiring

Use the following connections for the pin assignments described in the project code.

| Component | Connect to |
| --- | --- |
| HC-05 TXD | Arduino pin 2 |
| HC-05 RXD | Arduino pin 3 through a voltage divider |
| HC-05 VCC | 5V only if supported by your HC-05 breakout board |
| HC-05 GND | Arduino GND |
| Base servo signal | Arduino pin 9 |
| Shoulder servo signal | Arduino pin 10 |
| Elbow servo signal | Arduino pin 11 |

Power note: Use a suitable regulated external 5V supply for the three servos. Connect the external supply GND to Arduino GND. Do not power three servos directly from the Arduino 5V pin. The HC-05 RX input is generally not 5V tolerant, so use a voltage divider between Arduino pin 3 and HC-05 RXD.

# 4. Upload the Arduino Program

Install Arduino IDE from https://www.arduino.cc/en/software.

Connect the Arduino Uno to the computer with a USB data cable.

Open Arduino IDE and choose File → Open.

Select the Robotic_Arm.ino file.

If needed, open Sketch → Include Library → Manage Libraries and install the Servo library.

Choose Tools → Board → Arduino AVR Boards → Arduino Uno.

Choose Tools → Port and select the COM port for your Arduino.

Click Verify (✓). If there are no errors, click Upload (→).

Wait for the message “Done uploading”.

# 5. Install the Android App

Transfer Welding Arm Control.apk to your Android phone.

Open the APK using the phone's file manager.

If Android asks, allow installation from that source in Settings.

Tap Install, then open Welding Arm Control.

# 6. Pair the Phone with HC-05

Power on the Arduino and HC-05 module.

On the phone, open Settings → Bluetooth and turn Bluetooth on.

Search for devices and select HC-05.

Enter the PIN, usually 1234.

Open the app and choose HC-05 or tap Connect if those options are shown.

The Arduino sketch uses 9600 baud for the HC-05 serial connection.

# Quick Start Summary

Wire the Arduino, HC-05 and servos → upload Robotic_Arm.ino → install the APK → pair the phone with HC-05 → connect from the app → test each movement button slowly.